# NOISE CANCELLATION
AGENDA

## INTRODUCTION

## TECHNOLOGY

## APPLICATIONS

## DEMONSTRATION

## SUMMARY

# NOISE CANCELLATION
INTRODUCTION

## NOISE REDUCTION TECHNIQUES

### MECHANICAL DESIGN
BALANCE
TRANSIENTS
RESONANCES

### PASSIVE ATTENUATION
ENERGY ABSORBING MATERIALS
COMPLIANT MOUNTS

### ACTIVE SYSTEMS: NOISE CANCELLATION

## VIDEO

## APPLICATIONS

## STATUS

# "NOISE CANCELLATION TECHNOLOGY IS NOW A PRACTICAL SOLUTION TO REAL WORLD PROBLEMS"

# NOISE CANCELLATION
ANTI-NOISE

# "ACTIVE NOISE CANCELLATION DOES NOT MASK THE NOISE, IT REMOVES A SIGNIFICANT PORTION OF THE NOISE FROM THE ENVIRONMENT"

# NOISE CANCELLATION
TECHNOLOGY

## NOISE

### BROAD BAND    -	TURBULENT OR RANDOM NOISE

### NARROW BAND -	TONAL OR HARMONIC NOISE

## PASSIVE NOISE CONTROL

### LOCALIZE VIBRATION AND ABSORB ENERGY

### EFFECTIVE AT HIGH FREQUENCIES

## ACTIVE CANCELLATION

### TWO BASIC APPROACHES

BROAD BAND -	ADAPTIVELY FILTER THE SIGNAL FROM
 			AN "UPSTREAM" SENSOR TO CREATE
			THE ANTI-NOISE SIGNAL

SYNCHRONOUS - ADAPTIVELY SYNTHESIZE AN
			ANTI-NOISE SIGNAL WITH A-PRIORI 
			KNOWLEDGE OF THE BASIC "NOISE
			CYCLE" RATE

### EFFECTIVE AT LOW FREQUENCIES

# "A GOOD SOLUTION USUALLY REQUIRES A COMBINATION OF PASSIVE AND ACTIVE TECHNIQUES"

# NOISE CANCELLATION
EXAMPLE APPLICATIONS

## AT SOURCE

### MUFFLER

## ALONG A TRANSMISSION PATH

### IN-DUCT FAN NOISE

### COMPLIANT MOUNT

## AT THE EAR

### HEADSET

### QUIET ZONE
CABIN QUIETING
SILENT SEAT

# "ENGINEERING SOLUTIONS TO NOISE PROBLEMS REQUIRES AS MUCH REAL WORLD APPLICATION KNOWLEDGE AS IT DOES KNOWLEDGE OF ACTIVE NOISE CANCELLATION TECHNOLOGY"

# NOISE CANCELLATION
APPLICATION 1

# AT SOURCE: MUFFLER
(NARROW BAND NOISE CANCELLATION)

# "IT IS USUALLY BEST TO CANCEL THE NOISE AS CLOSE AS POSSIBLE TO THE SOURCE"

# NOISE CANCELLATION

# AT SOURCE:  MUFFLER
(CONTINUED)

## CHARACTERISTICS

### STRAIGHT THROUGH PASSIVE MUFFLER REDUCES BACKGROUND AND HIGH FREQUENCY NOISE

### ACTIVE CANCELLATION AT THE END OF THE EXHAUST GLOBALLY REDUCES TONAL NOISE COMPONENTS TO THE BACKGROUND

## BENEFITS

### LOW BACK PRESSURE

### FUEL ECONOMY

### QUIET

### SIZE AND WEIGHT

## APPLICATIONS

### PRIMARY POWER EXHAUST

### AUXILIARY POWER GENERATORS (EXHAUST NOISE)

### BULK TRANSFER VACUUM SYSTEMS

### COMPRESSOR INLET NOISE

# NOISE CANCELLATION
APPLICATION 2

# "FEED FORWARD CANCELLATION IS THE CLASSICAL APPROACH AND IS OFTEN HIGHLIGHTED IN THE TECHNICAL LITERATURE"

# NOISE CANCELLATION
APPLICATION 3

# "CONTROLLING VIBRATION NEAR THE SOURCE CAN GREATLY REDUCE ACOUSTICAL NOISE"

# NOISE CANCELLATION

# ALONG A TRANSMISSION PATH: COMPLIANT MOUNT
(CONTINUED)

## CHARACTERISTICS

### ROTATING MACHINE FLOATS FREELY AT VIBRATION FREQUENCIES

### NO VIBRATION ENERGY IS TRANSMITTED THROUGH THE MOUNT

### FAIL SAFE - THE PASSIVE MOUNT IS STILL THERE IF THE ACTIVE SYSTEM STOPS WORKING

## BENEFITS

### OPERATOR COMFORT - VIBRATION REDUCED

### SECONDARY ACOUSTICAL NOISE ELIMINATED

## APPLICATIONS

### EARTH MOVERS
ENGINES
AUXILIARY GENERATORS
REFRIGERATION UNITS
COMPRESSORS

### STATIONARY ROTATING MACHINES

# NOISE CANCELLATION
APPLICATION 4

AT THE EAR: HEADSET
(NARROW BAND NOISE CANCELLATION)

# "A SELECTIVE NOISE CANCELING HEADSET CAN IMPROVE WORKER SAFETY BY ENHANCING PERCEPTION OF WARNING SIGNALS"

# NOISE CANCELLATION

# AT THE EAR: HEADSET
(CONTINUED)

## CHARACTERISTICS

### SELECTIVE

### REDUCE TONAL COMPONENTS TO THE BACKGROUND
NOISE LEVEL

## BENEFITS

### SAFETY

### HEARING PROTECTION

### COMFORT

## APPLICATIONS

### EQUIPMENT OPERATORS (IN-CAB)

### GENERAL WORK SITE

# NOISE CANCELLATION
APPLICATION 5

# "PLACEMENT OF SPEAKERS AND MICROPHONES REQUIRES CAREFUL ACOUSTICAL ENGINEERING TO PRODUCE A WELL BEHAVED SYSTEM"

# NOISE CANCELLATION

# AT THE EAR: ZONE
(CONTINUED)

## CHARACTERISTICS

### EFFECTIVE ON TONAL NOISE BELOW 250 HZ

### SOPHISTICATED ALGORITHM COMPENSATES FOR INTERACTIONS AMONG MICROPHONES AND
SPEAKERS

## BENEFITS

### COMFORT

### SAFETY

### HEARING PROTECTION

## APPLICATIONS

### EQUIPMENT CAB

### SITE OFFICE

# NOISE CANCELLATION
APPLICATION 6

AT THE EAR: SILENT SEAT

# "A LOCALIZED QUIET ZONE CAN BE A COST EFFECTIVE SOLUTION FOR A WORKSTATION IN A NOISY ENVIRONMENT"

# NOISE CANCELLATION

# AT THE EAR: SILENT SEAT
(CONTINUED)

## CHARACTERISTICS

### EFFECTIVE FOR TONAL NOISE BELOW 250 HZ

### CONTROLLED INTERACTIONS BETWEEN THE CHANNELS

### LOCALIZED QUIET ZONE

## BENEFITS

### COST EFFECTIVE

### COMFORT

### SAFETY

### HEARING PROTECTION

## APPLICATIONS

### EQUIPMENT CAB

### SITE OFFICE

# NOISE CANCELLATION APPLICATION 1
(DEMONSTRATION TAPE)

## APPLICATION:	A 450 HORSEPOWER DIESEL ENGINE
			USED TO GENERATE ELECTRICAL POWER
			(DETROIT DIESEL 6V- 92 AT 1800 RPM)

## NOISE SOURCES

### EXHAUST	   -	USE AN ACTIVE MUFFLER, REMAINING
		SOURCES	CONTROLLED PASSIVELY

### AIR INTAKE -	AIR FILTER

### BLOCK	   -	PASSIVE MATERIALS IN HOUSING

### VIBRATION -	COMPLIANT MOUNTS

## RESULTS - EXHAUST REMOTED FROM ENGINE

| CONDITION | NOISE | BACK PRESSURE | NOTE |
| --- | --- | --- | --- |
| STRAIGHT PIPE | 123 dBA | NONE | 2 METERS FROM EXHAUST |
| CONVENTIONAL MUFFLER | 89 dBA | 1.5 INCHES HG | 12 FT LONG 2 FT DIAMETER |
| ACTIVE SYSTEM | 81 dBA | NONE | 2% FUEL SAVINGS 20% SMALLER |

# "THE ACTIVE MUFFLER SYSTEM WAS 20% SMALLER THAN THE CONVENTIONAL PASSIVE MUFFLER IT REPLACED"

# NOISE CANCELLATION APPLICATION 2
(SLIDES)

## APPLICATION:	A 100 HORSEPOWER DIESEL BLOWER
			USED TO LOAD AND UNLOAD RAILROAD CARS
			(CSX MASTERVAC - IN THE FIELD SINCE 10/90)

## NOISE SOURCES

### VACUUM EXHAUST -	USE AN ACTIVE MUFFLER, ALONG WITH A GOOD STRAIGHT THROUGH PASSIVE MUFFLER

### DIESEL EXHAUST    -	USE A BETTER PASSIVE MUFFLER

### BLOCK  -	PASSIVE MATERIALS IN HOUSING

### VIBRATION  -	COMPLIANT MOUNTS

# RESULTS (AT OPERATOR'S POSITION)

| CONDITION | NOISE | BACK PRESSURE | NOTE |
| --- | --- | --- | --- |
| ORIGINAL DESIGN | 115 dBA | 3 INCHES OF MERCURY | BLOWER OPERATES AT 15 INCHES OF MERCURY |
| ACTIVE SYSTEM | 91 dBA | NONE | 24% FUEL SAVINGS 20% FASTER |

# "THE QUIET SYSTEM LOADS/UNLOADS A RAIL CAR 20% FASTER THAN THE CONVENTIONAL NOISY SYSTEM IT REPLACED"

# NOISE CANCELLATION APPLICATION 3
(ACTIVE ANTI-VIBRATION DEMONSTRATION)

# "ACTIVE ANTI-VIBRATION TECHNOLOGY CAN BE USED IN A VARIETY OF APPLICATIONS TO IMPROVE MACHINE EFFICIENCY, REDUCE WEAR AND CONTROL VIBRATION-INDUCED NOISE."

# NOISE CANCELLATION
SUMMARY

## PRACTICAL SOLUTION TO SELECTED REAL WORLD PROBLEMS

## SIGNIFICANTLY REDUCES NOISE AND VIBRATION

## COSTS ARE OFTEN COVERED BY IMPROVED EFFICIENCIES

### SAFETY
ACCIDENTS
HEARING PROTECTION

### PRODUCTIVITY

### FUEL